DDC CODE DOWNLOADING PROCESS

The DDC programming is done at S-Wave Systems. The sequence followed in using the programming software is given below.

Connect one USB port of the PC to the DTCL USB through USB cable end terminated with Type C connectors

Connect another USB port to the STM32Cube programmer part no: ST-LINK/V2 through USB cable end terminated with Type c A connector on the PC side and Mini- B USB connector on the programmer side.

Connect the programmer output to the programming terminations on the DTCL

The connection diagram is shown in Figure 1

| DTCL MAIN PCB J6 |  |  | ST-LINK/V2 J4 |  |
| --- | --- | --- | --- | --- |
| SIGNL NAME | PIN NO |  | SIGNL NAME | PIN NO |
| JTCK_SWCLK | 2 |  | SWCLK | 9 |
| JTMS_SWDIO | 3 |  | SWDIO | 7 |
| NRST | 4 |  | NRST | 15 |
| GND | 5 |  | GND | 20,18 |

Indicate the connections on the berg pins on card

Locate STM32Cube programmer

On selecting the following screen will appear

Under ST-LINK configuration, go to dropdown button “Shared”

Under ST-LINK configuration, against “Shared” select enable,

Enabled will appear

Go to serial number and select the round arrow

Serial number will appear as shown, select connect button

The connected state will appear as shown

Select open file button

Browse DDC.elf file in the GUI installation path.

The browsed file will appear as shown, Select download button

Download will get initiated

On completion of download pop window will appear indication “file download complete” Select disconnect

Removing programming connection from the DTCL.